Chapter:10. Elasticity

1.Introduction: Elasticity is the property of a material by which it regains its original shape and size after the removal of the deforming force.

Elaboration: When an external force acts on a body, it undergoes deformation, which may involve a change in shape, size, or both. If the material returns completely to its original configuration after the removal of the force, the deformation is said to be elastic. This property is extremely important in engineering, construction, and material science because it determines how materials behave under load.

Example: Rubber band, steel spring, bow

Formula: No formula

Unit: No unit

Key Points:

- Works only within elastic limit

- Deformation is temporary and reversible

- Depends on nature, temperature, and structure of material

2. Interatomic Forces

Definition: Interatomic forces are the forces acting between atoms within a molecule or solid structure.

Elaboration: These forces consist of attractive forces (which pull atoms together) and repulsive forces (which push atoms apart). At equilibrium distance, these forces balance each other. When atoms are displaced due to an external force, restoring forces arise due to these interatomic interactions, which are responsible for elasticity in solids.

Example: Forces between atoms in a crystal lattice

Formula: F = -dU/dr

Where:

F = force between atoms

U = potential energy

R = distance between atoms

Unit: Newton (N)

Key Points:

- Strong forces compared to intermolecular forces

- Responsible for restoring force and elasticity

- Depend on atomic spacing and bonding

3. Intermolecular Forces

Definition: Intermolecular forces are the forces acting between molecules of a substance.

Elaboration: These forces are generally weaker than interatomic forces and include Van der Waals forces, dipole-dipole forces, and hydrogen bonding. They play an important role in determining physical properties such as elasticity in soft materials, viscosity, and surface tension.

Example: Forces between water molecules

Formula: Formula: F = -dU/dr

Where:

F = force between atoms

U = potential energy

R = distance between atoms

Unit: Newton (N)

Key Points:

- Weaker than interatomic forces

- Significant in liquids and gases

- Influence deformation and flexibility

4. Elastic Behaviour of Solids

Definition: Elastic behaviour refers to the ability of solids to return to their original shape and size after the removal of an applied force.

Elaboration: In solids, atoms are arranged in a well-defined structure. When an external force is applied, atoms are displaced slightly from their equilibrium positions. Due to interatomic forces, a restoring force is generated, which brings the atoms back to their original positions once the force is removed.

Example: Stretching and releasing a metal wire or spring

Formula: No formula

Unit: No unit

Key Points:

- Depends on atomic arrangement

- Limited by elastic limit

- More prominent in solids than fluids

5. Elasticity

Definition: Elasticity is the inherent property of a material to regain its original configuration after deformation.

Elaboration: Elasticity depends mainly on the strength of interatomic bonding. Materials with strong bonding (like steel) show greater elasticity compared to materials like rubber when measured in terms of restoring force. Elasticity is essential in designing structures, machines, and instruments.

Example: Steel spring used in machines

Formula: No formula

Unit: No unit

Key Points:

- Opposite of plasticity

- Related to stress and strain

- Essential for engineering applications

6. Perfectly Elastic Body

Definition: A perfectly elastic body is one that completely regains its original shape and size after the removal of deforming force for all values within elastic limit.

Elaboration: Such a body strictly obeys Hooke’s law for all applied stresses within the elastic range. In practice, no material is perfectly elastic, but some materials approximate this behavior under small deformations.

Example: Ideal spring (theoretical concept)

Key Points:

- No permanent deformation

- No loss of energy

- Ideal or hypothetical concept

7. Perfectly Plastic Body

Definition: A perfectly plastic body is one that does not regain its original shape after the removal of deforming force.

Elaboration: In such materials, once deformation occurs, it remains permanently even after removing the force. These materials are useful in shaping and molding processes.

Example: Clay, wax, putty

Key Points:

- Permanent deformation occurs

- No restoring force

- Does not obey Hooke’s law

8. Stress

Definition: Stress is defined as the internal restoring force per unit area developed within a body when an external force is applied.

Elaboration: When a body is subjected to external force, internal forces are generated within the material to resist deformation. These internal forces per unit area constitute stress.

Example: Force applied on a wire or rod

Formula: Stress = F / A

Unit: Pascal (Pa)

Key Points:

- Internal quantity

- Opposes applied force

- Can be tensile, compressive, or shear

9. Types of Stress

(a) Normal Stress

Definition: Stress acting perpendicular to the surface of the material.

(i) Tensile Stress: Produced when a body is stretched by pulling forces, resulting in increase in length.

(ii) Compressive Stress: Produced when a body is compressed by pushing forces, resulting in decrease in length.

(b) Shear Stress (Tangential Stress)

Definition: Stress acting parallel to the surface, causing layers to slide over one another.

Formula: Shear Stress = F / A

Example: Cutting paper with scissors

(c) Bulk Stress

Definition: Stress that produces a change in volume without change in shape.

Example: Pressure applied to a submerged object

Key Points:

- Different stresses cause different deformations

- Important in structural analysis

- Measured in Pascal

10. Strain

Definition: Strain is the ratio of change in dimension to the original dimension of a body.

Elaboration: Strain measures how much deformation has occurred in a material relative to its original size. It is a dimensionless quantity.

Example: Change in length of a stretched wire

Formula: Strain = ΔL / L

Unit: No unit (dimensionless)

Key Points:

- No unit

- Indicates degree of deformation

- Proportional to stress within elastic limit

11. Types of Strain

(a) Longitudinal Strain: Ratio of change in length to original length

(b) Volumetric Strain: Ratio of change in volume to original volume

(c) Shear Strain: Angular deformation caused by shear stress

Key Points:

- Different strains correspond to different stresses

- Important in elasticity calculations

- Dimensionless quantities

12. Elastic Limit

Definition: Elastic limit is the maximum value of stress up to which a material returns to its original shape after removal of force.

Elaboration: If the applied stress exceeds this limit, the material undergoes permanent deformation and does not regain its original configuration.

Key Points:

- Boundary of elastic behavior

- Important for safe design of structures

- Depends on material properties

13. Hooke’s Law

Definition: Within elastic limit, stress is directly proportional to strain.

Formula: σ = Eε

Elaboration: This law forms the foundation of elasticity. The constant of proportionality (E) is called Young’s modulus and represents the stiffness of the material.

Key Points:

- Valid only within elastic limit

- Linear relationship

- Basis for defining elastic constants

14.Experimental Verification of Hooke’s Law

Definition: Hooke’s law states that within the elastic limit, stress is directly proportional to strain.

Elaboration: To verify Hooke’s law experimentally, a wire or spring is suspended vertically and different loads are added gradually. The extension produced in the wire is measured using a scale. It is observed that the extension increases proportionally with the applied force. A graph is plotted between load (force) and extension. The graph is a straight line passing through the origin, proving that force ∝ extension. This verifies Hooke’s law within the elastic limit.

Example: A spring stretches by 2 cm when a force of 10 N is applied. When the force is increased to 20 N, the extension becomes 4 cm, showing direct proportionality.

Formula:

F ∝ x

F = kx

Or

Stress ∝ Strain

Stress = Y × Strain

Where:

F = applied force

X = extension

K = spring constant

Y = Young’s modulus

Unit:

Force (F) = Newton (N)

Extension (x) = meter (m)

Spring constant (k) = N/m

Stress = Pascal (Pa)

Strain = no unit

Key Points:

1. Hooke’s law is valid only within the elastic limit.

2. The force–extension graph is a straight line through the origin.

3. Beyond elastic limit, the material shows permanent deformation.

15. Variation of Strain with Stress(Stress-Strain Curve)

Definition: A graphical representation of the relationship between stress and strain.

Elaboration: The curve consists of different regions such as proportional limit, elastic limit, yield point, plastic region, and breaking point. It helps in determining the strength, ductility, and toughness of materials.

Key Points:

- Shows behavior under load

- Identifies elastic and plastic regions

- Important for engineering design

16. Types of Moduli of Elasticity

Definition: Elastic moduli are the ratios of stress to corresponding strain, representing the stiffness of a material.

(a) Young’s Modulus (Y)

Formula:

Y = Longitudinal Stress / Longitudinal Strain

Unit: Pascal

Example: Stretching of a wire

(b) Bulk Modulus (K)

Formula:

K = Bulk Stress / Volumetric Strain

Example: Compression of liquids

(c) Shear Modulus (G)

Formula:G = Shear Stress / Shear Strain

Key Points:

- Measure rigidity of materials

- Higher value means more stiffness

- Important in engineering calculations

17.Young’s Modulus of Elasticity (Y)

Definition: Young’s modulus is the ratio of longitudinal stress to longitudinal strain within the elastic limit.

Elaboration: When a wire or rod is stretched by applying a force, its length increases. Young’s modulus measures how much the material resists this change in length. A higher value of Y means the material is more rigid and less stretchable.

Example: Steel has a high Young’s modulus, so it stretches very little under force, while rubber has a low Young’s modulus and stretches easily.

Formula: Y = Stress / Strain

= (F/A) / (ΔL/L)

= FL / (AΔL)

Unit: Pascal (Pa) = N/m²

Q.YOUNG’S MODULUS OF ELASTICITY (Y)

Statement: Young’s Modulus of Elasticity is defined as the ratio of longitudinal stress to longitudinal strain within the elastic limit of a material.

Formula:

Y = (Longitudinal Stress) / (Longitudinal Strain)

Derivation: Consider a uniform wire of:

Original length = L

Cross-sectional area = A

Diagram:

F →  ┌───────────────┐  ← F

│     Wire      │

-- L --

fig:(I)

After applying force:

F →  ┌───────────────────┐  ← F

│       Wire        │

---- L + ΔL ---

fig:(II)

A force F is applied along the length of the wire.

Due to this force:

Extension produced = ΔL

1.Longitudinal Stress:

Stress = Force / Area

Stress = F / A

2.Longitudinal Strain:

Strain = Change in length / Original length

Strain = ΔL / L

3.Young’s Modulus:

Y = Stress / Strain

Substituting values:

Y = (F/A) / (ΔL/L)

Y = (F × L) / (A × ΔL)

Hence,

Y = (F L) / (A ΔL)

Key Points:

1. Measures resistance to change in length.

2. Valid only within elastic limit.

3. Higher Y → more rigid material.

18.Bulk Modulus of Elasticity (K)

Definition: Bulk modulus is the ratio of volumetric stress (pressure) to volumetric strain.

Elaboration: When a body is subjected to uniform pressure from all sides, its volume decreases. Bulk modulus measures the resistance of a material to change in volume. Materials with high K are difficult to compress.

Example: Liquids and solids have high bulk modulus, while gases have very low bulk modulus.

Formula:

K = -Pressure / (ΔV/V)

= -P / (ΔV/V)

Unit: Pascal (Pa)

Q.BULK MODULUS OF ELASTICITY (K)

Statement: Bulk Modulus of Elasticity is defined as the ratio of bulk stress to volumetric strain within the elastic limit of a material.

Formula:

K = (Bulk Stress) / (Volumetric Strain)

Derivation: Consider a body of:

Original volume = V

Diagram:

Before Pressure:

┌───────────┐

│           │

│   Body    │   Volume = V

│           │

└───────────┘

fig:(I)

After Applying Pressure:

↓   ↓   ↓

┌───────────┐

→  │           │  ←

│   Body    │   Volume = V - ΔV

→  │           │  ←

└───────────┘

↑   ↑   ↑

fig:(II)

When a uniform pressure P is applied on all sides,

The volume changes by ΔV.

1.Bulk Stress:

Bulk stress = Pressure applied

Bulk stress = P = F / A

2.Volumetric Strain:

Volumetric strain = Change in volume / Original volume

Volumetric strain = ΔV / V

3.Bulk Modulus:

K = Bulk stress / Volumetric strain

K = P / (ΔV / V)

K = (P × V) / ΔV

Since volume decreases under pressure,

ΔV is negative.

So,

K = - (P × V) / ΔV

Hence,

K = - PV / ΔV

Key Points:

1. Measures resistance to change in volume.

2. Negative sign shows decrease in volume under pressure.

3. Higher K → less compressible material.

19.Shear Modulus (G) or Modulus of Rigidity

Definition: Shear modulus is the ratio of shear stress to shear strain within the elastic limit.

Elaboration: When a tangential force is applied, the shape of the body changes without changing its volume. Shear modulus measures the resistance to this change in shape.

Example: When pushing the top surface of a book sideways while the bottom remains fixed, the book deforms due to shear stress.

Formula:

G = Shear Stress / Shear Strain

= (F/A) / θ

Unit: Pascal (Pa)

Q.SHEAR MODULUS OR MODULUS OF RIGIDITY (G)

Statement: Shear Modulus (or Modulus of Rigidity) is defined as the ratio of shear stress to shear strain within the elastic limit of a material.

Formula:

G = (Shear Stress) / (Shear Strain)

Derivation: Consider a cube of side L fixed at the bottom.

Diagram:

Before applying force:

┌───────────┐

│           │

│   Cube    │

│           │

└───────────┘

fig:(I)

After applying tangential force:

┌───────────┐ → F

/           /

/   Cube    /

/           /

└───────────┘

fig:(II)

(Top surface shifts by Δx forming angle θ)

A tangential force F is applied on the top surface,

Causing a lateral displacement Δx.

Let,

Area of top surface = A = L²

1.Shear Stress:

Shear stress = Force / Area

Shear stress = F / A

2.Shear Strain:

Shear strain = Lateral displacement / Height

Shear strain = Δx / L

(Also, shear strain ≈ angle θ for small deformation)

3. Shear Modulus:

G = Shear stress / Shear strain

Substituting values:

G = (F/A) / (Δx/L)

G = (F × L) / (A × Δx)

Hence,

G = (F L) / (A Δx)

Key Points:

1. Measures resistance to change in shape.

2. No change in volume during shear deformation.

3. Higher G → more rigid against shear force.

20. Measurement of Young’s Modulus of Elasticity, Y

Definition: The experimental determination of Young’s modulus of a material.

Formula: Y = FL / (AΔL)

Elaboration: It is commonly measured using Searle’s apparatus by applying load to a wire and measuring its extension accurately.

Q.MEASUREMENT OF YOUNG’S MODULUS (Y)

Statement: Young’s modulus of a material is defined as the ratio of longitudinal stress to longitudinal strain

Within the elastic limit.

Apparatus: Searle’s apparatus, wire, scale, micrometer screw gauge, weights.

Diagram:

Rigid Support

─────────────

Reference Experimental

Wire      Wire

Scale      Pointer

|

Weight (W)

fig:(I)

Description: Two identical wires are suspended from a rigid support. One is a reference wire, and the other is

An experimental wire. A load is applied to the experimental wire, causing extension, which is

Measured using a scale and pointer arrangement.

Derivation: Let,

L = original length of the wire

A = cross-sectional area of the wire

R = radius of the wire

ΔL = extension produced

F = applied force (F = mg)

Stress = Force / Area

= F / A

Strain = Extension / Original length

= ΔL / L

Young’s Modulus (Y) = Stress / Strain

Y = (F / A) ÷ (ΔL / L)

Y = (F / A) × (L / ΔL)

Y = FL / (AΔL)

Since, A = πr²

Y = FL / (πr²ΔL)

Final formula: Y = FL / (πr²ΔL)

Key Points:

- Requires precise measurement

- Depends on temperature and material

- Common laboratory experiment

21.Energy Stored in a Stretched Wire

Definition: The work done in stretching a wire is stored as elastic potential energy.

Elaboration: When a force is applied gradually, energy is stored in the material due to deformation. This energy can be recovered when the force is removed.

Formula: U = 1/2 Fx

Unit: Joule (J)

Q.ENERGY STORED IN A STRETCHED WIRE

Statement: The energy stored in a stretched wire is equal to the work done in stretching it within the elastic limit.

Diagram:

Initial State                Stretched State

──────────────              ──────────────

|            |              |            |

|    Wire    |              |   Wire     |

|            |              |            |

──────────────              ──────────────

↓ F

(Force)

fig:(I)

Derivation: When a wire is stretched by applying a force, it undergoes extension. Work is done against the

Restoring force of the wire, and this work is stored as elastic potential energy.

Let,

L = original length of wire

A = cross-sectional area

Y = Young’s modulus

ΔL = extension produced

F = applied force

From Young’s modulus:

Y = (F/A) / (ΔL/L)

F = Y A (ΔL / L)

As the force increases gradually from 0 to F,

Average force = F/2

Work done = Average force × extension

W = (F/2) × ΔL

Substitute F:

W = (1/2) × [Y A (ΔL / L)] × ΔL

W = (1/2) × (Y A / L) × (ΔL)²

Energy Stored(U):

U = (1/2) × (Y A / L) × (ΔL)²

Alternate form:

Since strain = ΔL / L

U = (1/2) × Stress × Strain × Volume

U = (1/2) × Y × (strain)² × Volume

Final formula:

U = (1/2) × (Y A / L) × (ΔL)²

Key Points:

- Recoverable energy

- Depends on force and extension

- Used in springs and elastic systems

22. Poisson’s Ratio

Definition: Poisson’s ratio is the ratio of lateral strain to longitudinal strain.

Elaboration: When a material is stretched, it contracts in the direction perpendicular to the applied force. This ratio quantifies that contraction.

Formula:

μ = Lateral strain / Longitudinal strain

Unit: No unit

Q.POISSON’S RATIO(DERIVATION)

Statement: Poisson’s ratio is defined as the ratio of lateral strain to longitudinal strain within the elastic limit.

Diagram:

Before Stretching            After Stretching

───────────────             ───────────────

|             |             |           |

|   Wire      |   -      |    Wire   |

|             |             |           |

───────────────             ───────────────

Length = L                 Length = L + ΔL

Diameter = D               Diameter = D – ΔD

fig:(I)

Derivation: When a wire is stretched, its length increases while its diameter decreases.

The ratio of decrease in diameter to increase in length gives Poisson’s ratio.

Let,

L = original length

ΔL = increase in length

D = original diameter

ΔD = decrease in diameter

Longitudinal strain = ΔL / L

Lateral strain = ΔD / D

Poisson’s Ratio (μ):

Μ = Lateral strain / Longitudinal strain

Μ = (ΔD / D) / (ΔL / L)

Μ = (ΔD × L) / (D × ΔL)

NOTE:

Since ΔD represents decrease, sometimes a negative sign is used:

Μ = - (lateral strain / longitudinal strain)

Final formula: Μ = (ΔD / D) / (ΔL / L)

Key Points:

- Value ranges between 0 and 0.5

- Important material property

- Used in structural design

23.Elastic After Effect

Definition: Elastic after effect is the delay in a material regaining its original shape after the removal of stress.

Elaboration: In some materials, recovery is not instantaneous and takes time due to internal friction and molecular rearrangement.

Example: Rubber slowly returning to its original shape

Key Points:

- Time-dependent phenomenon

- Seen in viscoelastic materials

- Indicates non-ideal elasticity

24. Elastic Fatigue

Definition: Elastic fatigue is the gradual loss of elasticity of a material due to repeated loading and unloading.

Elaboration: Repeated stress cycles weaken the material and may eventually lead to fracture even if the stress is below the elastic limit.

Example: Metal wire breaking after repeated bending

Key Points:

- Causes material failure

- Important in machine parts

- Depends on number of cycles

25. Elastic Hysteresis

Definition: Elastic hysteresis is the phenomenon in which some energy is lost as heat during a complete cycle of loading and unloading.

Elaboration: In a stress-strain graph, the loading and unloading paths form a loop. The area enclosed by this loop represents the energy lost during the cycle.

Example: Rubber band showing heat loss after repeated stretching

Key Points:

- Represents energy dissipation

- Important in damping systems

- Occurs in cyclic deformation

FORMULA SHEETS

1.Stress = Force / Area

Σ = F / A

2.Strain = Change in dimension / Original dimension

Ε = ΔL / L

3.Young’s Modulus

Y = Stress / Strain

4.Y in terms of force

Y = FL / (AΔL)

5.Bulk Modulus

K = - (P / (ΔV / V))

6.Shear Modulus

Η = Shear stress / Shear strain

7.Shear Modulus (basic)

Η = F / (Aθ)

8.Relation between Y, K, η

Y = 9Kη / (3K + η)

9.Relation with Poisson’s ratio

Y = 2η(1 + μ)

10.Bulk modulus relation

K = Y / [3(1 – 2μ)]

11.Shear modulus relation

Η = Y / [2(1 + μ)]

12.Poisson’s ratio

Μ = (lateral strain) / (longitudinal strain)

13.Energy stored in stretched wire

U = (1/2)(F × ΔL)

14.Energy using Young’s modulus

U = (1/2)(YA/L)(ΔL)²

15.Energy per unit volume

U = (1/2) × stress × strain

16.Work done

W = (1/2)FΔL

17.Hooke’s Law

F ∝ ΔL  or  F = kΔL

18.Spring constant

K = F / ΔL

19.Young’s modulus in terms of k

Y = (kL) / A

20.Pressure = Force / Area

P = F / A

21.Volume strain

Εv = ΔV / V

22.Longitudinal strain

Εl = ΔL / L

23.Lateral strain

Εlat = ΔD / D

24.Stress in terms of Y

Stress = Y × Strain

25.Elastic potential energy (general)

U = (1/2) kx²

NUMERICAL

NEB, ADVANCED LEVEL(2073 – 2082)

1.(Composite Wire Extension)

Two wires of same material, lengths L1 = 1 m, L2 = 2 m and same area are connected in series.

Force applied = 200 N, Y = 2×10¹¹ N/m², A = 1×10⁻⁶ m².

Find total extension.

Solution:

ΔL = FL/AY

Total extension:

ΔL = F/A × (L1/Y + L2/Y)

ΔL = (200 / 1×10⁻⁶) × (3 / 2×10¹¹)

ΔL = 2×10⁸ × (1.5×10⁻¹¹)

ΔL = 3×10⁻³ m

2.(Parallel Wire System)

Two wires of same length but areas A and 2A are connected in parallel.

Total force = 300 N.

Find extension.

Solution:

Force distributes proportional to area:

F1:F2 = A:2A = 1:2

F1 = 100 N, F2 = 200 N

Extension same:

ΔL = FL/AY

Using total:

Equivalent area = 3A

ΔL = FL / (3AY)

3.(Energy Stored – Variable Force)

A wire obeys Hooke’s law up to 100 N and extends 2 mm.

Find total work done.

Solution:

U = (1/2)FΔL

U = (1/2) × 100 × 2×10⁻³

U = 0.1 J

4.(Breaking Stress)

A wire breaks under force 400 N. Radius = 1 mm.

Find breaking stress.

Solution:

A = πr² = π(10⁻³)² = π×10⁻⁶

Stress = F/A

= 400 / (π×10⁻⁶)

≈ 1.27×10⁸ N/m²

5.(Energy per Unit Volume)

Stress = 10⁸ N/m², Y = 2×10¹¹

Find energy density.

Solution:

U = (1/2)(stress² / Y)

U = (1/2)(10⁸)² / (2×10¹¹)

U = 2.5×10⁴ J/m³

6.(Bulk Compression Work)

Volume = 0.02 m³, pressure = 5×10⁶ N/m²

Volume decreases by 0.2%

Solution:

ΔV = 0.2% = 0.002 × 0.02 = 4×10⁻⁵

Work:

W = (1/2)PΔV

W = (1/2)(5×10⁶)(4×10⁻⁵)

W = 100 J

7.(Wire Under Own Weight)

Length = 2 m, density = 8000 kg/m³, Y = 2×10¹¹

Find extension due to its own weight.

Solution:

ΔL = (ρgL²) / (2Y)

= (8000×9.8×4) / (2×2×10¹¹)

= 1.568×10⁻⁶ m

8.(Relation of Moduli – Advanced)

Y = 2×10¹¹, K = 1.6×10¹¹

Find Poisson’s ratio.

Solution:

Y = 3K(1 – 2μ)

2×10¹¹ = 3(1.6×10¹¹)(1 – 2μ)

Solve:

Μ = 0.25

9.(Tapering Wire – Conceptual)

A wire tapers such that radius varies. If volume constant, show extension increases.

Result:

ΔL ∝ 1/A (smaller area → more extension)

10.(Maximum Energy Storage)

A wire of breaking stress σ can store maximum energy:

Umax = (σ² / 2Y) × Volume

Ssolution:

Given:

Σ = 2×10⁸, Y = 2×10¹¹

Umax/volume:

= (4×10¹⁶) / (4×10¹¹)

= 10⁵ J/m³

11.Two wires of lengths 1 m and 2 m connected in series. Force 200 N, same area and Y. Find total extension.

Solution:

ΔL = F/A × (L1/Y + L2/Y)

= (200 /1×10⁻⁶)(3 /2×10¹¹)

= 3×10⁻³ m

12.Two wires in parallel with areas A and 2A carry total 300 N. Find extension.

Solution:

Equivalent area = 3A

ΔL = FL/(3AY)

13. A wire breaks under 500 N. Radius = 1 mm. Find breaking stress.

Solution:

A = π(10⁻³)² = π×10⁻⁶

Stress = 500/(π×10⁻⁶) ≈ 1.59×10⁸ N/m²

14.Stress = 10⁸ N/m², Y = 2×10¹¹. Find energy density.

Solution:

u = (1/2)(stress²/Y) = (1/2)(10⁸)²/(2×10¹¹) = 2.5×10⁴ J/m³

15.A body of volume 0.01 m³ under pressure 4×10⁶ N/m² shrinks by 0.1%. Find work done.

Solution:

ΔV = 0.001×0.01 = 1×10⁻⁵

W = (1/2)PΔV = (1/2)(4×10⁶)(1×10⁻⁵) = 20 J

16.A wire of length 2 m, density 8000 kg/m³, Y = 2×10¹¹. Find extension due to its own weight.

Solution:

ΔL = (ρgL²)/(2Y) = (8000×9.8×4)/(4×10¹¹) = 1.568×10⁻⁶ m

17.Y = 2×10¹¹, K = 1.6×10¹¹. Find Poisson’s ratio.

Solution:

Y = 3K(1−2μ)

2×10¹¹ = 4.8×10¹¹(1−2μ)

μ = 0.25

18.Maximum energy per unit volume if breaking stress = 2×10⁸ and Y = 2×10¹¹.

Solution:

u = σ²/(2Y) = (4×10¹⁶)/(4×10¹¹) = 10⁵ J/m³

19.A spring of constant 1000 N/m stretches 2 cm. Find energy stored.

Solution:

U = (1/2)kx² = (1/2)(1000)(0.02)² = 0.2 J

20.A rod of area 1×10⁻⁴ m² under force 1000 N. Find stress.

Solution:

Stress = F/A = 1000/(1×10⁻⁴) = 1×10⁷ N/m²

SHORT QUESTIONS AND ANSWERS

1.What is elasticity?

Ans: Elasticity is the property of a material by virtue of which it regains its original shape and size after the removal of deforming force.

2. What is stress?

Ans: Stress is the restoring force per unit area developed inside a body when an external force is applied.

Formula: Stress = Force / Area

3. What is strain?

Ans: Strain is the ratio of change in dimension to the original dimension of a body.

Formula: Strain = Change in dimension / Original dimension

4. State Hooke’s law.

Ans: Within the elastic limit, stress is directly proportional to strain.

i.e., Stress ∝ Strain

5. What is Young’s modulus?

Ans: Young’s modulus is the ratio of longitudinal stress to longitudinal strain.

Y = (Longitudinal stress) / (Longitudinal strain)

6. What is bulk modulus?

Ans: Bulk modulus is the ratio of volumetric stress to volumetric strain.

K = (Pressure) / (Volumetric strain)

7. What is shear modulus?

Ans: Shear modulus is the ratio of shear stress to shear strain.

G = (Shear stress) / (Shear strain)

8. Define elastic limit.

Ans: Elastic limit is the maximum stress up to which a material returns to its original shape after removal of force.

9. What is plasticity?

Ans: Plasticity is the property of a material by which it does not regain its original shape after deformation.

10. What is breaking stress?

Ans: Breaking stress is the stress at which a material breaks.

11. Define factor of safety.

Ans: Factor of safety is the ratio of ultimate stress to working stress.

FOS = Ultimate stress / Working stress

12. What is Poisson’s ratio?

Ans: It is the ratio of lateral strain to longitudinal strain.

Poisson’s ratio = Lateral strain / Longitudinal strain

13. Define longitudinal strain.

Ans: It is the ratio of change in length to the original length.

14. Define volumetric strain.

Ans: It is the ratio of change in volume to the original volume.

15. Define shear strain.

Ans: Shear strain is the angular deformation produced due to shear stress.

16. What is a perfectly elastic body?

Ans: A body that completely regains its original shape after removing force is called perfectly elastic.

17. What is a perfectly plastic body?

Ans: A body that does not regain its original shape at all is called perfectly plastic.

18. State relation between elastic constants.

Ans: Y = 2G(1 + μ)

19. Why steel is preferred over rubber for springs?

Ans: Because steel has high Young’s modulus and returns quickly to its original shape.

20. What is strain energy?

Ans: The energy stored in a body due to deformation is called strain energy.

MULTIPLE CHOICE QUESTIONS

1. The property by which a body regains its original shape after deformation is called:

A. Plasticity

B. Elasticity

C. Viscosity

D. Rigidity

Ans: B

2. Stress is defined as:

A. Force × Area

B. Force / Area

C. Area / Force

D. Force × Distance

Ans: B

3. The SI unit of stress is:

A. N

B. Pa

C. J

D. W

Ans: B

4. Strain is:

A. Dimensionless

B. Has unit meter

C. Has unit Newton

D. Has unit Pascal

Ans: A

5. Hooke’s law is valid up to:

A. Breaking point

B. Elastic limit

C. Yield point

D. Ultimate point

Ans: B

6. Young’s modulus is defined as:

A. Stress / Strain

B. Strain / Stress

C. Force / Length

D. Energy / Volume

Ans: A

7. Bulk modulus is related to:

A. Length change

B. Volume change

C. Shape change

D. Temperature change

Ans: B

8. Shear modulus is associated with:

A. Volume change

B. Shape change

C. Length change

D. Mass change

Ans: B

9. Poisson’s ratio is the ratio of:

A. Longitudinal strain / Lateral strain

B. Lateral strain / Longitudinal strain

C. Stress / Strain

D. Pressure / Volume

Ans: B

10. Which of the following has maximum elasticity?

A. Rubber

B. Steel

C. Glass

D. Plastic

Ans: B

11. The unit of Young’s modulus is:

A. N

B. Pa

C. m

D. kg

Ans: B

12. If stress is doubled within elastic limit, strain:

A. Doubles

B. Halves

C. Remains same

D. Becomes zero

Ans: A

13. Factor of safety is:

A. Working stress / Ultimate stress

B. Ultimate stress / Working stress

C. Stress / Strain

D. Strain / Stress

Ans: B

14. The curve between stress and strain is straight line up to:

A. Breaking point

B. Elastic limit

C. Yield point

D. Ultimate point

Ans: B

15. Which material is most elastic?

A. Rubber

B. Steel

C. Clay

D. Wax

Ans: B

16. Dimensional formula of stress is:

A. ML⁻¹T⁻²

B. ML⁻²T⁻²

C. ML²T⁻²

D. MLT⁻²

Ans: B

17. Bulk modulus is defined as:

A. Longitudinal stress / strain

B. Pressure / volumetric strain

C. Shear stress / strain

D. Force / area

Ans: B

18. Shear strain is measured in:

A. Radian

B. Meter

C. Pascal

D. Newton

Ans: A

19. Energy stored due to deformation is:

A. Kinetic energy

B. Potential energy

C. Strain energy

D. Heat energy

Ans: C

20. Relation between elastic constants:

A. Y = 2G(1 + μ)

B. Y = G(1 + μ)

C. Y = 3G(1 – μ)

D. Y = G – μ

Ans: A

21. A perfectly elastic body:

A. Never deforms

B. Fully regains shape

C. Breaks easily

D. Expands permanently

Ans: B

22. Plasticity means:

A. Complete recovery

B. No deformation

C. Permanent deformation

D. Elastic recovery

Ans: C

23. Which has highest bulk modulus?

A. Gas

B. Liquid

C. Solid

D. Vacuum

Ans: C

24. Strain depends on:

A. Material

B. Force only

C. Dimensions

D. None

Ans: D

25. Stress-strain curve slope gives:

A. Strain

B. Stress

C. Young’s modulus

D. Energy

Ans: C